Removal Efficiency

Average Daily Removal Efficiency (ADRE)

Rpotw = Σ[(Ir,n – Epotw,n)/Ir,n]

N

Rpotw  	=  Plant Removal Efficiency			decimal%

Ir  	=  Average Influent Pollutant Concentration	mg/l

Epotw	=  Average Effluent Pollutant Concentration	mg/l

n            =  Paired data sets, numbered 1 to N                     integer

Mean Removal Efficiency (MRE)

Rpotw  =  Ir – Epotw

Ir

Rpotw  	=  Plant Removal Efficiency			decimal%

Ir  	=  Average Influent Pollutant Concentration	mg/l

Epotw	=  Average Effluent Pollutant Concentration	mg/l

Mean Removal Efficiency from Biosolids Data

Rpotw  =  (Su  x  8.34  x  PS/100  x  Qbiosolids  x  Gbiosolids)

(Ir  x  8.34  x  Qpotw)

Rpotw  		=  Plant Removal Efficiency				decimal%

PS		=  Average Percent Solids of Biosolids to Disposal	%

Su		=  Average Biosolids Pollutant Concentration		mg/kg

Qbiosolids	=  Average Total Biosolids Daily Flow Rate to Disposal	MGD

Gbiosolids	=  Average Specific Gravity of Biosolids (water = 1.0)	kg/l

Ir  		=  Average Influent Pollutant Concentration		mg/l

Qpotw		=  POTW Average Daily Flow Rate			MGD

Allowable/Maximum Allowable Headworks Loading

AHL/MAHL based on MEPDES Permit Limit

AHLmepdes  =  8.34  x  Cmepdes  x  Qpotw

(1  -  Rpotw)

AHLmepdes	=  AHL/MAHL based on MEPDES Permit Limit	#/day

Cmepdes	=  MEPDES Permit Limit (convert from #/day at	mg/l

average daily flow)

Qpotw	  	=  POTW Average Daily Flow Rate		MGD

Rpotw  		=  Plant Removal Efficiency			decimal%

AHL/MAHL based on Acute, Chronic, of Human Health Criteria

AHLwq  =  8.34  x  [Cwq  (Qstr  +  Qpotw)  -  (Cstr  x  Qstr)]

(1  -  Rpotw)

AHLwq	=  AHL/MAHL based on Acute, Chronic, or Human Health Water Quality Criteria	#/day

Cwq	=  State Water Quality Criteria, current Chapter 584, Appendix A			mg/l

Qstr	=  Receiving Stream (upstream) Flow						MGD

(Utilize the appropriate dilution factor flows: acute/modified acute, chronic,

and harmonic mean, respectively, from Fact Sheet. 1 cfs = 0.6463 MGD)

Qpotw	=  POTW Average Daily Flow Rate						MGD

Cstr	=  Receiving Stream background pollutant concentration (10% of Water		mg/l

Quality Criteria)

Rpotw  	=  Plant Removal Efficiency							decimal%

AHL/MAHL based on Secondary Inhibition Pollutant Concentration

AHL2°inh  =  8.34  x  C2°inh  x  Qpotw

1  -  R2°inh

AHL2°inh	=  AHL/MAHL based on Secondary Inhibition Pollutant Concentration	#/day

C2°inh		=  Secondary Treatment Inhibition Pollutant Concentration (from	mg/l

EPA Local Limits Development Guidance Appendices, 2004,

Appendix G – Literature Inhibition Values, or site-specific data)

Qpotw		=  POTW Average Daily Flow Rate					MGD

R2°inh		=  Removal Efficiency through Primary Treatment (from EPA Local	decimal%

Limits Development Guidance Appendices, 2004, Appendix G -

Priority Pollutant Removal Efficiencies, or site specific data)

AHL/MAHL based on Biosolids Criteria

AHLbiosolids  =  (Su  x  8.34  x  PS/100  x  Qbiosolids  x  Gbiosolids)

(Ir  x  8.34  x  Qpotw)

AHLbiosolids  	=  AHL/MAHL based on Biosolids			#/day

PS		=  Average Percent Solids of biosolids to Disposal	%

Su		=  Average Biosolids Pollutant Concentration		mg/kg

Qbiosolids	=  Average Total Biosolids Daily Flow Rate to Disposal	MGD

Gbiosolids	=  Average Specific Gravity of Biosolids			kg/l

Ir  		=  Average Influent Pollutant Concentration		mg/l

Qpotw		=  POTW Average Daily Flow Rate			MGD

Maximum Allowable Industrial Headworks Loading

Residential/Commercial Loading Determination

Lunc  =  8.34  x  Cunc  x  Qunc

Lunc	=  Headworks Loading from Uncontrolled Residential and Commercial Sources	#/day

Cunc	=  Pollutant Concentration of Uncontrolled Residential and Commercial Sources	mg/l

Qunc	=  Flow from Uncontrolled Residential and Commercial Sources			MGD

MAIHL

MAIHL  =  [MAHL  x  (1  -  SF)]  -  [Lunc  +  HW  +  GA]

MAIHL	=  Maximum Allowable Industrial Headworks Loading					#/day

MAHL	=  Maximum Allowable Headworks Loading (the most restrictive of the calculated AHLs)	#/day

SF	=  Safety Factor (this is optional; recommend a minimum of .05)				decimal%

Lunc	=  Headworks Loading from Uncontrolled Residential and Commercial Sources		#/day

HW	=  Loading from hauled waste, not covered by IPT Program				#/day

GA	=  Growth Allowance (this is optional, based on 5 years’ anticipated future development)	#/day

Uniform Concentration Limit

Uniform Concentration Limit

Cucl  =            MAIHL____

8.34  x  Qindustrial

Cucl		=  Uniform Concentration Limit						mg/l

Qindustrial	=  Total Daily Flow from all Industrial Contributors			MGD